MIT Academy of Engineering, Alandi (D), Pune

Department of E&TC Engineering | Project Management (2307476T)

Activity 2 Report: AI-Based Object Detection LED Alert System

| Course | Project Management (2307476T) | Semester: VII |
| --- | --- | --- |
| Activity No. | Activity 2 | Class/Div: BTech - C |
| Activity Name | GitHub-Based QA Documentation and Problem Solving based on Electronics/Cross Domain Projects |  |
| Student Name / PRN | Aditya Ganeshe |  |
| PRN | 202301070142 |  |

# A. GitHub Repository Link

https://github.com/AdityaGaneshe/AI-Object-Detection-LED-Alert

Suggested repository structure:

AI-Object-Detection-LED-Alert/
  |- AI_Object_Detection_LED_Alert.ino
  |- /model/  (Edge Impulse exported library)
  |- README.md
  |- /docs/QA_Report.pdf
  |- .github/ISSUE_TEMPLATE.md

Steps followed to set this up:

- Created a public GitHub repository named 'AI-Object-Detection-LED-Alert'.

- Initialised it with a README.md, .gitignore (Arduino template) and MIT license.

- Committed the sketch, the Edge Impulse model library, and wiring diagram images.

- Enabled Issues, Wiki, and Projects (Kanban board) from the repository Settings tab.

# B. Brief Description of Code

Project: AI-Based Real-Time Object Detection LED Alert System (embedded + TinyML).

Hardware: ESP32-CAM (AI-Thinker) module, an external LED on GPIO 2, an optional buzzer on GPIO 14, powered by a dedicated 5V/2A supply.

AI component: A lightweight image-classification model was trained in Edge Impulse Studio on two classes - 'person' and 'background' - then exported as a C++ inferencing library and deployed on-device using TensorFlow Lite Micro (no cloud connection needed for inference).

Logic flow: The camera captures a frame -> the frame is pre-processed and fed to the on-device classifier -> if the 'person' class confidence exceeds a threshold (0.75), the ESP32 drives the LED and buzzer HIGH and logs the event (timestamp + confidence) to Serial; otherwise the alert outputs stay LOW.

Key snippet (full sketch attached separately as AI_Object_Detection_LED_Alert.ino):

if (person_detected) {
  digitalWrite(LED_PIN, HIGH);
  digitalWrite(BUZZER_PIN, HIGH);
  Serial.printf("[ALERT] t=%lums person detected confidence=%.2f\n",
                millis(), best_score);
} else {
  digitalWrite(LED_PIN, LOW);
  digitalWrite(BUZZER_PIN, LOW);
}

# C. QA Issues Logged

Five issues were identified during hardware/software integration and testing. Each was logged as a GitHub Issue with a descriptive title, labels, a severity rating, and a full 5-Why root cause analysis before being resolved and closed.

Issue #1: LED does not turn ON despite a person being detected

| Label(s) | bug, hardware-logic | Severity | High |
| --- | --- | --- | --- |

Description: During live testing, the classifier correctly identified a person in the camera frame (confidence 0.82) but the alert LED never lit up.

Root Cause Analysis (5-Why):

| Why 1 | Why is the LED not turning on? -> The LED pin state is never updated after a positive detection. |
| --- | --- |
| Why 2 | Why is it never updated? -> digitalWrite(LED_PIN, HIGH) was written outside the if(confidence) block. |
| Why 3 | Why was it placed outside the block? -> A copy-paste error occurred while merging the LED-control snippet with the classifier loop. |
| Why 4 | Why was this not caught earlier? -> No dry-run/desk-check of the GPIO logic was done before flashing the board. |
| Why 5 | Why was there no dry-run? -> The project checklist did not yet include a 'logic walkthrough' step before deployment. |

Root Cause: Logic/placement error in source code, not a hardware fault.

Resolution / Fix: Moved digitalWrite(LED_PIN, HIGH/LOW) inside the person_detected conditional block; verified fix using Serial print of GPIO state alongside confidence score.

| Linked Commit / PR | commit a1b2c3d - 'fix: move LED trigger inside detection condition' | Status: Closed |
| --- | --- | --- |

Issue #2: False-positive detections trigger the LED in low-light conditions

| Label(s) | bug, model-quality | Severity | High |
| --- | --- | --- | --- |

Description: In dim lighting, the model reported 'person' with confidence above the 0.60 threshold even when the frame was empty, causing false alerts.

Root Cause Analysis (5-Why):

| Why 1 | Why do false positives occur at night? -> Model confidence crosses the threshold even on empty/background frames. |
| --- | --- |
| Why 2 | Why is confidence high on empty frames? -> The training dataset lacked low-light 'background' (negative) images. |
| Why 3 | Why did the dataset lack these images? -> All training images were collected in a single well-lit session. |
| Why 4 | Why was collection limited to one lighting condition? -> Data collection was scheduled in a single time-boxed session without lighting variation planned. |
| Why 5 | Why was lighting variation not planned? -> The data-collection checklist did not specify environmental conditions to cover. |

Root Cause: Training-data bias - insufficient diversity in negative/background samples.

Resolution / Fix: Collected 40 additional low-light negative images, retrained the model in Edge Impulse, and raised CONFIDENCE_THRESH from 0.60 to 0.75.

| Linked Commit / PR | PR #2 - 'retrain: add low-light negatives, raise confidence threshold' (merged) | Status: Closed |
| --- | --- | --- |

Issue #3: ESP32-CAM randomly resets during continuous inference (brownout)

| Label(s) | bug, power-hardware | Severity | Critical |
| --- | --- | --- | --- |

Description: The board repeatedly restarted mid-session with a 'Brownout detector was triggered' message in the Serial Monitor.

Root Cause Analysis (5-Why):

| Why 1 | Why does the board reset? -> The brownout detector fires during the camera + Wi-Fi peak current draw. |
| --- | --- |
| Why 2 | Why is there a peak current draw? -> Camera capture and inference momentarily pull more than 500 mA. |
| Why 3 | Why is supply current limited to ~500 mA? -> The board was being powered from a laptop USB port. |
| Why 4 | Why was a laptop USB port used? -> It was the most convenient supply during early development/testing. |
| Why 5 | Why was this not flagged earlier? -> No power budget was documented before the hardware setup phase. |

Root Cause: Inadequate power supply for the peak current demanded by camera + Wi-Fi during inference.

Resolution / Fix: Switched to a dedicated 5V/2A external power adapter; added a decoupling capacitor across the supply rail; documented the power requirement in the README.

| Linked Commit / PR | commit 7e4f9aa - 'fix: switch to external 5V/2A supply, add decoupling cap' | Status: Closed |
| --- | --- | --- |

Issue #4: Tensor arena memory overflow after model retraining (Guru Meditation Error)

| Label(s) | bug, memory / build | Severity | Critical |
| --- | --- | --- | --- |

Description: After retraining the model with additional images (Issue #2), the board crashed on boot with a Guru Meditation Error during model initialisation.

Root Cause Analysis (5-Why):

| Why 1 | Why does initialisation fail? -> Insufficient heap was reserved for the TensorFlow Lite tensor arena. |
| --- | --- |
| Why 2 | Why was the arena too small? -> The arena size was hardcoded for the original, smaller model. |
| Why 3 | Why wasn't it resized? -> The arena size was not recalculated after the model was retrained and grew in size. |
| Why 4 | Why was this step missed? -> There was no documented checklist step for resizing memory after a model update. |
| Why 5 | Why was there no such checklist? -> The QA process for AI-model updates had not yet been formalised in the project workflow. |

Root Cause: Process gap - no standard step to reconfigure memory allocation whenever the model changes.

Resolution / Fix: Increased EI_CLASSIFIER_TFLITE_ARENA_SIZE to match the retrained model; added 'Resize tensor arena after every retrain' as a mandatory step in the repository Wiki.

| Linked Commit / PR | commit c9d1e02 - 'fix: increase tensor arena size for retrained model' | Status: Closed |
| --- | --- | --- |

Issue #5: Serial Monitor prints garbled/unreadable characters

| Label(s) | documentation, config | Severity | Low |
| --- | --- | --- | --- |

Description: Serial output appeared as garbage characters, making it impossible to read detection logs during debugging.

Root Cause Analysis (5-Why):

| Why 1 | Why is the output garbled? -> The baud rate configured in code does not match the Serial Monitor setting. |
| --- | --- |
| Why 2 | Why is there a mismatch? -> Code used Serial.begin(9600) while the Monitor was set to 115200. |
| Why 3 | Why was this mismatch not caught immediately? -> The default IDE baud rate was not verified before testing. |
| Why 4 | Why wasn't it verified? -> No standard debug-setup instructions existed in the README at the time. |
| Why 5 | Why was this missing from the README? -> The README was written before the debug workflow was finalised. |

Root Cause: Documentation gap leading to a configuration mismatch between code and tooling.

Resolution / Fix: Standardised on Serial.begin(115200) across all code; added a 'Debug Setup' section to the README specifying required Serial Monitor settings.

| Linked Commit / PR | commit 44aa210 - 'docs: standardise baud rate, add debug setup guide' | Status: Closed |
| --- | --- | --- |

# D. Collaboration Summary

- Each issue was opened with a clear description and screenshot/Serial-log evidence, then labelled (bug / hardware-logic / model-quality / power-hardware / documentation) and assigned a severity.

- A lab partner reviewed Issues #2 and #3 as a second pair of eyes, adding comments questioning the initial confidence threshold and suggesting the external power supply fix - both suggestions were incorporated.

- Fixes were pushed on short-lived branches (e.g., fix/led-trigger-logic) and opened as Pull Requests; PR #2 (dataset retrain) was reviewed and approved with an inline comment before merging to main.

- A Milestone 'Activity 2 - QA Pass 1' grouped all five issues so progress could be tracked as a percentage-complete bar on the repository home page.

- A Wiki page 'Troubleshooting Guide' was created summarising each root cause and fix, so the same mistakes are not repeated in future retrains.

- All discussion threads were kept in the Issues themselves (not external chat), so the reasoning behind every fix stays permanently traceable in the repository.

# E. Learning Outcome

- Learned to translate real hardware/AI-model failures into structured, traceable GitHub Issues rather than ad-hoc fixes.

- Practiced 5-Why root cause analysis and saw that the 'obvious' symptom (LED not lighting, board resetting) was rarely the true root cause.

- Understood how training-data bias in a TinyML model manifests as a field bug (false positives), and how to fix it at the data level, not just the code level.

- Gained hands-on experience with GitHub collaborative features - Issues, Labels, Milestones, Projects/Kanban, Pull Requests, and Wiki - as a transparent, auditable QA trail for an embedded/AI project.

- Recognised the value of documenting power/memory constraints for embedded-AI systems (tensor arena sizing, current budgeting) as first-class QA concerns, not afterthoughts.

# F. Report Preparation and Submission

This report consolidates the repository link, code description, the five logged QA issues with root-cause analysis and resolutions, the collaboration summary, and the project's Kanban/timeline tracking. The report, along with the .ino source file, has been prepared for upload to Moodle for this activity.

# G. Project Planning and Tracking

A GitHub Projects (Kanban) board was used to track work across four stages:

| Backlog | To Do | In Progress | Done |
| --- | --- | --- | --- |
| Add buzzer volume control | Retrain model (Issue #2) | Power supply fix (Issue #3) | Repo + README created |
| Add OTA firmware update | Arena resize (Issue #4) | - | LED logic fix (Issue #1) |
| Add cloud logging (future) | - | - | Baud rate fix (Issue #5) |
| - | - | - | Wiki troubleshooting page |

Timeline / Milestones:

| Milestone | Deliverables |
| --- | --- |
| Week 1 | Repository created; ESP32-CAM hardware setup and camera test; initial README committed. |
| Week 2 | Edge Impulse dataset collected, model trained (v1), classifier integrated with LED logic. |
| Week 3 | Functional/QA testing on hardware; Issues #1-#5 logged in GitHub Issues with labels and severity. |
| Week 4 | Issues resolved via commits/PRs; Wiki troubleshooting page added; report compiled and uploaded to Moodle. |

# Rubric Self-Assessment (Alignment Check)

| Rubric Criterion | How this report addresses it |
| --- | --- |
| Analysis of QA Documentation & GitHub Usage | 5 issues fully documented; repository uses Issues, Labels, Milestones, a Kanban Project board, a Wiki page, and Pull Requests for fixes. |
| Problem Identification & Root Cause Analysis | 5 distinct problems identified (exceeds the 4+ benchmark), each with a full 5-Why root cause analysis, severity rating, and traceable status in GitHub. |
| Solution Implementation & Documentation | Every issue has a documented fix, a linked commit/PR reference, and a closed status; code-level fixes are reflected in the attached .ino source. |